HW1—Exact motion planning

Q2

Prove A⊕(B∪C) = (A⊕B)∪(A⊕C):

s ∈A⊕(B∪C)  ∃ q1,q2 :s=q1+q2 , q1∈A  ∧q2∈ B∪C

∃ q1,q2 :s=q1+q2 , q1∈A  ∧(q2∈ B or q2∈ C)

∃ q1,q2 :s=q1+q2 ,( q1∈A  ∧q2∈ B) or ( q1∈A  ∧q2∈ C)

s∈ A⊕B or s∈ A⊕C

s∈ (A⊕B)∪(A⊕C)

What is the Minkowski sum of:

- Two points : a single point , Let the points be pand q then    {p}⊕{q}={p+q}.

- A point and a line: Let the point be p, and let Lbe a line,
{p}⊕L={p+ℓ:ℓ∈L}. meaning we got the same line shifted by vector p.

Two line segments:
Case 1: Segments are parallel:
we will get line segment whose length is the sum of the two segment lengths.
Case 2 : Segments are not parallel: we will get parallelogram.

- Two Disks:

A disk whose center is the sum of the centers and whose radius is the sum of the radii.

Prove "E ≤3V −6" (V ≥ 3)
if E=0,1,2 then the statement is true because we assume V>=3 (E<=2<3<=3V −6)
else( E>=3 ):

We will define face-degree=d(f)=number of edges that form f boundary

- Each face is bounded by at least 3 edges(that is true for the external face because E>=3) so:
d(f)≥3 for every face f ⇒ fd(f)≥3F.

- Every edge contributes exactly 2 to the total face-degree count (one per adjacent face), so:
fd(f)=2E

- From  the previous points we get : 2E≥3F.

- We saw in the class Euler’s formula V-E+F=2.
⇒F=2+E-V ⇒3F=6+3E-3V
combining with the last point:
6+3E-3V≤2E ⇒E≤3V-6

Q3

3.2

Result of the given input :

Result of new input :

Computational Complexity of the Implementation:

Let n be the number of vertices of the workspace obstacle P,
and let m be the number of vertices of the robot polygon R.
Our Minkowski sum implementation is based on the convex-hull method, which consists of two main steps:

1-Generating all pairwise point sums

for (px, py) in P:

for (qx, qy) in R:

sums.append((px + qx, py + qy))

Since we generate exactly n×mpoints, the time cost is: O(nm)

2- Computing the convex hull of S,
Using Shapely computing a convex hull for N=nm points takes:
O(Nlog(N))=O (nm log (nm))

In total, time complexity is O(nm)+O(nm log(nm))=O(nm log(nm))

How can non-convex obstacles affect the results?
it does not work for non-convex polygons because it removes the concavities of the actual Minkowski sum, resulting in a shape that is the convex hull of the true sum, not the sum itself. The algorithm only identifies the outermost boundary points and misses internal geometric features critical for non-convex shapes. So Using the convex hull algorithm for non-convex polygons produces an approximation that is larger than the actual sum .

non-convex obstacle example :

The figure demonstrates that when using the convex-hull Minkowski sum algorithm on a non-convex obstacle, the concave portion of the obstacle is removed,  producing a convex and larger c-space obstacle. This over-approximates the true Minkowski sum, preventing some valid paths.